/********************************** C80S.C *********************************

Copyright Symbios Logic Inc.

Author:		Sherri Moore Symbios Logic System Applications Engineer

Purpose:		53C80S Demonstration Software

This software was written by Symbios Logic to develop and test new

products.  It is not a general SCSI driver.  Symbios Logic assumes no liability for

its use, misuse or inability to use.

Project files:

C80S.C 		Main body of code

TARGET80.C	Target Mode routines

C80SISR.C	Interrupt Service Routine

C80STEST.C	Automatic Test Module

DMA80S.C	Hardware dependant DMA code (8237 compatible)

DMA80S.H  	DMA controller parameters

C80S_AT.H	Hardware addresses, global defines

C80S_EV.H	External variables

C80SCAM.C	SCAM Menu

C80SCAM.H

rev 1.0	  7-95	Initial release as C80SDEMO

****************************************************************************/

#include <stdio.h>

#include <conio.h>

#include <stdlib.h>

#include <dos.h>

#include <process.h>

#include <ctype.h>

#include <string.h>

#include "c80s_at.h"

/***************************** Global Variables **************************/

struct parms {  unsigned 	mid, tid, atn, msg_byte, blksize, blkcount,

timeout, parity, detail, DMA, ena_eop,

tar_mode, scsi_reset, clr_reset;

} parms;

struct SCAMparms {

unsigned char 	target, seldelay, priority, assigned_id[8];

char 		idstring[32];

} SCAMparms;

int 		i, j, temp1, temp2, page, pagemax;

unsigned long 	seg, off, abs_adrs, iocount, byte_count;

unsigned int 	dma_buf[16384], cdb_buf[9], sence_data[7], intstat, reg_set,

msg, x_count, scsicmd, statbyte, inquiry_data[35];

unsigned char 	far* cdb, far* dmabyte, scsi_error, cmd_error, mode, group_code,

bus_ID, ID_msg, status, cur_data, scsi_stat, initiat,

tar_phase, cdb_bytes, rcv_byte, pause_flag, TCR;

char 		cmd,temp,looprw,setup_cmd;

/*********************************** main **********************************

inputs:  none

outputs: stdout CRT

calls:   almost everything

Description:  This module prints a menu on the standard output (called CRT

through out this program) and prompts the user to enter a letter from the

menu corresponding to some test to perform.  All of the tests are performed

in other modules.

***************************************************************************/

void main(void)

{

init ();					/* Init variables, chips, motherboard DMA, */

while (1)

{       do						/* clear unexpected interrupts */

{	status = inportb(stat_reg);

if (status & 0x10)

{

inportb(rst_int);				/*clear interrupts*/

printf("\n Clear Interrupt.");

status = inportb(stat_reg);

if (status == 0xFF)

{printf("\nCHIP NOT RESPONDING\n");

return;

}

}

printf("INTERRUPT ");

}while (status & 0x10);

clrscr();

scsi_error = false;

printf("\n\n DMA start address = %06lX",abs_adrs); 		/* troubleshooting */

printf("\n\n    ********************* Symbios 53C80S AT Board Test Menu********************\n");

printf("\n  b .................... Display buffer memory");

printf("\n  d .................... Init buffer with decrementing pattern");

printf("\n  e .................... Execute an SCSI command");

printf("\n  i .....................Init buffer with incrementing pattern");

printf("\n  j .....................Fill buffer with alternating 00 FF");

printf("\n  l .....................Loop on read or write");

printf("\n  m .....................SCAM Menu");

printf("\n  r .....................Read 53C80S Registers");

printf("\n  R .....................Reset SCSI bus");

printf("\n  s .....................Setup menu");

printf("\n  t .....................Target mode");

printf("\n  q .................... Quit; exit this program");

printf("\n\n ? ");

cmd=getche();

switch (cmd)

{case 'b' :

dumpbuf();			/* display buffer memory */

break;

case 'd' :				/* decrement buffer, start at keyboard input*/

printf("\n Initialize memory buffer with an decrementing pattern");

printf("\n Enter starting value in hex ? ");

scanf("%x%c",&j,&temp);	/* temp is to discard carriage return */

fillbuff(cmd,j);

break;

case 'e' :					/* Execute an SCSI command */

scsi_menu();

if(cmd_error)

printf("\n   Unsupported Command !");

break;

case 'i' :				/* increment buffer, start at keyboard input */

printf("\n Initialize memory buffer with an incrementing pattern");

printf("\n Enter starting value in hex ? ");

scanf("%x%c",&j,&temp);	/* temp is to discard carriage return */

fillbuff(cmd,j);

break;

case 'j' :				/* init buffer with alternating 0C 0D hex */

fillbuff(cmd,0);

break;

case 'l' :

clrscr();

printf("\n\nLoop Read/Write Test\n\n");

looprw = 'r';

load_regs();

cdb[1]=0x00;

cdb[2]=0x00;

cdb[3]=0x00;

cdb[4]=parms.blkcount;

cdb[5]=0x00;

do {

if (looprw == 'r')

looprw = 'w';

else

looprw = 'r';

if (looprw == 'r')

cdb[0]=0x08;	/* SCSI Read Command */

else

cdb[0] =0x0a;

DATA_DRIVER(looprw, 6, byte_count);

} while(!kbhit() && !scsi_error);

break;

case 'm' :					/* SCAM menu */

SCAM_menu();

pause_flag = false;

break;

case 'q' :

return;					/* quit program */

case 'r' :

read_regs();			/* read 53CF92 registers */

break;

case 'R' :					/* SCSI Reset */

outportb(init_cmd, 0x80);

outportb(init_cmd, 0x00);			init();

break;

case 's' :

setup();				/* parameter Set-up menu */

break;

case 't' :

tar_phase = 0x00;

target_mode();

if (scsi_error) {

printf("\n\nhit any key to continue");

getch();

}

break;

default:

printf("\nInvalid response.  Enter a case sensitive letter from the menu.\n");

delay(1500);

}					/* end of switch function  */

if(scsi_error)

{outportb(init_cmd, 0x80);		/* SCSI RST/ */

outportb(init_cmd, 0x00);

load_regs();

}

if (pause_flag == true) 		/* pause to view results on CRT */

{printf("\nhit any key to return to main menu");

temp = getch();

pause_flag=false;

}

}

}							/* end of main */

/******************************* Initialize *********************************

inputs:  none

outputs: none

calls:   initIBMdma(), load_regs(), init86C01() *microchannel version only*

*************************************************************************/

void init(void) {

dmabyte = (unsigned char *) dma_buf;	/* align char array to word boudary */

cdb     = (unsigned char *) cdb_buf;

parms.mid = 0x02;				/* my ID, SCSI ID 7 */

parms.tid = 0x01;				/* destination target device ID 0 */

parms.atn = 'y';				/* Select without ATN */

parms.msg_byte =0x80;				/* ID Message Byte = 0 */

parms.blksize = 512;				/* number of bytes in data block */

parms.blkcount = 1; 				/* number of blocks to transfer */

parms.detail = 'y';				/* Display lots of info during SCSI */

parms.parity = 'y';				/* enable automatic parity checking  & int*/

parms.DMA = 'y';				/* use DMA for data phase */

parms.ena_eop = 'y';				/* Enable EOP Interrupt*/

parms.tar_mode = 'n';				/* Target Mode*/

parms.scsi_reset = 0x80;			/* Set Enable RST/ */

parms.clr_reset = 0x00;				/* Clear Enable RST/ */

SCAMparms.target = 1;				/* SCAM Tolerant target device */

SCAMparms.seldelay = 0x06;			/* selection response delay of 10ms */

SCAMparms.priority = 1;				/* priority flag for SCAM isolation */

stpcpy(&SCAMparms.idstring[2],"Symbios ");		/* Vendor ID Code */

stpcpy(&SCAMparms.idstring[10],"Slave Device 1");	/* Vendor Unique Code */

outportb(init_cmd, 0x80);

outportb(init_cmd, 0);		/* Reset */

load_regs();

initIBMdma(dmabyte, byte_count, 'w');

}

/**************** Load Registers in 53C80S SCSI Controller ***************/

void load_regs(void)

{byte_count = parms.blksize * parms.blkcount;	/* number bytes to send */

initiat = 0;

outportb(init_cmd, initiat);

mode = 0;

outportb(mod_reg, mode);

tar_phase = 0;

outportb(tgt_cmd, tar_phase);

}

/******************************** fill buffer *******************************

fillbuff(pattern, start)

input arguemnt:	start   = starting value of pattern

pattern = type of pattern to fill the memory buffer with.

i = incrementing pattern starting at keyboard input.

d = decrementing pattern starting at keyboard input.

j = alternating 00 FF (hex).

**************************************************************************/

void fillbuff(unsigned char pattern, unsigned int start) {

byte_count = parms.blksize * parms.blkcount;

switch (pattern) {

case 'i' :

for (i=0; i<byte_count; ++i)

dmabyte[i] = start + i;

break;

case 'd' :

for (i=0; i<byte_count; ++i)

dmabyte[i] = start - i;

break;

case 'j' :

for (i=0; i<byte_count; i=i+2)

dmabyte[i] = 0x00;

for (i=1; i<byte_count; i=i+2)

dmabyte[i] = 0xFF;

}								/* end of switch statement */

}									/* end of fillbuf() */

/******************************* dump buffer ********************************

dumpbuf()

This routine prints the memory array out to the CRT.  There are a varible

number of pages, each of which has 16 rows of 16 bytes displayed in hex.

***************/

void dumpbuf(void) {

pagemax = byte_count >> 8;			/* total-bytes divided by bytes-per-page */

page = 0;

clrscr();

do {

printf("\n\n");

for (j = 0; j < 16; j++) {

printf("\n");

for (i = 0; i < 16; i++)

printf("%02X  ",dmabyte[i + (j<<4) + (page<<8)]);

}							/* end of j loop */

++page;

printf("\n\n   %d pages left.  ", pagemax - page);

printf("\n space bar to continue or q to quit");

temp = tolower(getch());

} while (temp != 'q' &&  page < pagemax);

}									/* end of dumpbuf */

/************************** Parameter Set-Up menu **************************/

void setup(void) {

do {

clrscr();

printf("\n\n     **************************** Set-Up Menu ****************************\n");

printf("\n  a - select with ATN ....................    %c",parms.atn);

printf("\n  b - block size (bytes per block)........ %4d",parms.blksize);

printf("\n  B - block count (blocks to transfer).... %4d",parms.blkcount);

printf("\t  total bytes: %d",parms.blksize * parms.blkcount);

printf("\n  c - Check Parity .......................    %c",parms.parity);

printf("\n  D - DMA Transfers during Data Phase.....    %c",parms.DMA);

printf("\n  e - Enable EOP Interrupt................    %c",parms.ena_eop);

printf("\n  i - ID message byte ....................   %02Xh",parms.msg_byte);

printf("\n  m - My ID ..............................   %02Xh",parms.mid);

printf("\n  t - Target ID ..........................   %02Xh",parms.tid);

printf("\n  v - Detailed Messages to CRT ...........    %c",parms.detail);

printf("\n  q - Exit this menu");

printf("\n\n ? ");

setup_cmd = getche();

switch (setup_cmd) {

case 'a' :					/* set flag for select with or without Attention */

do {

printf("\n Select with Attention  (y/n) ?");

parms.atn = tolower(getche());

} while (parms.atn != 'y' && parms.atn != 'n');	/* accept only y or n */

if(parms.atn == 'y') {

printf("\n ID Message byte in hex ?");	/* need Message Byte */

scanf("%x%c", &parms.msg_byte, &temp);

}

break;

case 'b' :					/* change Block Size */

do {

printf("\n New Block Size in decimal (4096 max) ? ");

scanf("%d%c",&parms.blksize,&temp);

} while (parms.blksize > 16384);

/* no BREAK statement here to next question */

case 'B' :					/* change Block Count */

do {

printf("\n Block Size * Block Count = 16384 bytes maximum");

printf("\n New Block Count in decimal (%d max) ? ",16384/parms.blksize);

scanf("%d%c",&parms.blkcount,&temp);

} while (parms.blkcount * parms.blksize > 16384);

break;

case 'c' :

do {

printf("\n Enable automatic Parity Checking ? ");

parms.parity = tolower(getche());

} while (parms.parity != 'y' && parms.parity != 'n');

break;

case 'D' :

do {

printf("\n Use DMA during Data Phase ? ");

parms.DMA = tolower(getche());

} while (parms.DMA != 'y' && parms.DMA != 'n');

break;

case 'e' :

do {

printf("\n Enable EOP Interrupt ? ");

parms.ena_eop = tolower(getche());

} while (parms.ena_eop != 'y' && parms.ena_eop != 'n');

break;

case 'i' :					/* change ID Message Byte */

printf("\nnew ID Message Byte in hex ? ");

scanf("%x%c",&parms.msg_byte,&temp);

break;

case 'm' : 				/* change My ID */

do {

printf("\n New ID of this machine in hex ? ");

scanf("%02X%c",&parms.mid,&temp);

} while (parms.mid > 0x80);

break;

case 'q' :

break;				/* need a place holder to avoid default */

case 't' :					/* change Target ID */

do 	{

printf("\n New Target ID in hex ? ");

scanf("%02X%c",&parms.tid,&temp);		/* temp is to discard CRLF */

} while (parms.tid > 0x80);

break;

case 'v' :

printf("\n Verbose mode during SCSI (y/n) ? ");

temp = tolower(getche());

switch (temp)

{

case 'n' :

parms.detail = temp;

break;

case 'y' :

parms.detail = temp;

break;

default:

printf("\n\nDetail setting not changed");

} 					/* end of get character case statement */

break;

default:

printf("Invalid response.  Enter a case sensitive letter from the menu.\n");

delay(1500);

}							/* end of switch(setup_cmd) */

} while (setup_cmd != 'q');						/* end of do-while loop */

load_regs();								/* load the parameters into the chip */

while(!kbhit());

temp = getche();

}									/* end of setup() */

/******************************** SCSI menu ********************************/

void scsi_menu(void) {

scsi_error = false;

cmd_error = false;

scsicmd = 0;

clrscr();

printf("\n****************************** SCSI Command Menu *****************************\n");

printf("\n  00 ............................. Test Unit Ready");

printf("\n  03 ............................. Request Sense");

printf("\n  08 ............................. Read");

printf("\n  0a ............................. Write");

printf("\n  12 ............................. Inquiry");

printf("\n  28 ............................. Extended Read");

printf("\n  2a ............................. Extended Write");

printf("\n\n ? ");

scanf("%x",&scsicmd);

switch(scsicmd) {

case 0x00 :

tur();     					/* Test Unit Ready */

break;

case 0x03 :					/* Request Sense */

printf("\nRequest Sense  (display buffer memory to see sense data)\n");

cdb[0] = 0x03;				/* op-code */

cdb[1] = 0x00;				/* lun = 0 */

cdb[2] = 0x00;				/* reserved */

cdb[3] = 0x00;				/* reserved	*/

cdb[4] = 0xff;				/* allocation length */

cdb[5] = 0x00;				/* control byte */

rqsense(6);				/* driver for Request Sense */

break;

case 0x08 :        				/* SCSI read command */

cdb[0] = 0x08;

cdb[1] = 0x00;				/* lun = 0, msbaddr = 0	*/

cdb[2] = 0x00;				/* msb address = 00	*/

cdb[3] = 0x00;				/* lsb address = 00	*/

cdb[4] = parms.blkcount;			/* number blocks to transfer */

cdb[5] = 0x00;				/* reserved byte */

DATA_DRIVER ('r', 6, byte_count);	/* driver for data transfer*/

break;

case 0x01 :                     		/* SCSI read command-delay REQs */

cdb[0] = 0x08;

cdb[1] = 0x00;

cdb[2] = 0x01;

cdb[3] = 0x00;

cdb[4] = parms.blkcount;

cdb[5] = 0x01;              			/* code to add delay between REQs */

DATA_DRIVER('r',6,byte_count);

break;

case 0x0a :					/* SCSI write command */

cdb[0] = 0x0a;				/* opcode */

cdb[1] = 0x00;				/* lun = 0, msbaddr = 0	*/

cdb[2] = 0x00;				/* msb address = 00	*/

cdb[3] = 0x00;				/* lsb address = 00	*/

cdb[4] = parms.blkcount;			/* number blocks to transfer */

cdb[5] = 0x00;				/* reserved byte */

DATA_DRIVER ('w', 6, byte_count);	/* driver for data transfer*/

break;

case 0x12 :					/* SCSI inquiry command */

printf("\nInquiry Command\n");

inquiry();

break;

case 0x28 : 					/* Extended Read */

cdb[0] = 0x28;				/* Extended Read Command */

cdb[1] = 0x00;				/* lun = 0 */

cdb[2] = 0x00;				/* MSB of block address	*/

cdb[3] = 0x00;				/* 2SB of block address	*/

cdb[4] = 0x01;				/* 3SB of block address	*/

cdb[5] = 0x00;				/* LSB of block address	*/

cdb[6] = 0x00;				/* reserved byte */

cdb[7] = parms.blkcount>>8;		/* MSB of transfer length */

cdb[8] = parms.blkcount;		/* LSB of transfer length */

cdb[9] = 0x00;				/* control byte	*/

DATA_DRIVER ('r', 10, byte_count);	/* driver for data transfer*/

break;

case 0x2a : 					/* Extended Write Bench Mark Test */

cdb[0] = 0x2a;				/* Exteded Write Command */

cdb[1] = 0x00;				/* lun = 0 */

cdb[2] = 0x00;				/* MSB of block address	*/

cdb[3] = 0x00;				/* 2SB of block address	*/

cdb[4] = 0x01;				/* 3SB of block address	*/

cdb[5] = 0x00;				/* LSB of block address	*/

cdb[6] = 0x00;				/* reserved byte */

cdb[7] = parms.blkcount>>8;		/* MSB of transfer length */

cdb[8] = parms.blkcount;		/* LSB of transfer length */

cdb[9] = 0x00;				/* control byte	*/

DATA_DRIVER ('w', 10, byte_count); 	/* driver for data transfer*/

break;

default :

cmd_error = true;				/* unsupported command */

}						/* end of switch statement */

}							/* end of scsimenu */

/******************************* Test Unit Ready ************************/

void tur (void) {

pause_flag = true;

printf("\nTest Unit Ready Command\n");

cdb[0] = 0x00;				/* op-code */

cdb[1] = 0x00;				/* lun = 0 */

cdb[2] = 0x00;				/* reserved	*/

cdb[3] = 0x00;				/* reserved	*/

cdb[4] = 0x00;				/* reserved	*/

cdb[5] = 0x00;				/* control byte	*/

select(6);				/* select a device */

if (scsi_error)

return;

cmdcomp();				/* Get Status, Msg, accept & disconnect */

if (scsi_error)

return;				/* exit, abort */

while ((status & 0x03) || scsi_stat || cur_data)	/* Poll for Bus Free */

{scsi_stat = inportb(scsi_bus);

status = inportb(stat_reg);

cur_data  = inportb(scsi_data);

}

tar_phase = 0;

outportb(tgt_cmd, tar_phase);

}

/******************************* Inquiry Command ************************/

unsigned char inquiry(void)

{int i;

pause_flag = true;

cdb[0] = 0x12;					/* op-code */

cdb[1] = 0x00;					/* lun = 0 */

cdb[2] = 0x00;					/* page code */

cdb[3] = 0x00;					/* reserved	*/

cdb[4] = 0x24;					/* allocation length = 36 bytes */

cdb[5] = 0x00;					/* control byte	*/

select(6);

if (scsi_error)

return;

while (!(inportb(scsi_bus) & 0x20));	/* Poll for REQ/ active (valid Phase) */

scsi_stat = inportb(scsi_bus);

while ((scsi_stat & 0x1C) == 0x1C)		/* Message In ? */

{Message_In();				/* returns w/ valid phase */

if (scsi_error)

return(0);

}

if ((scsi_stat & 0x1C) != 0x04)         /* Data In Phase? */

{printf("\n Illegal Phase, expected Msg or Data In");

printf("\n Current SCSI Bus Status = %02X", inportb(scsi_bus));

scsi_error = true;

return(0);

}			  	/* Illegal Phase, abort */

printf("\n Inquiry Data In ...");

outportb(tgt_cmd, 1);			/* Set PC bits to Data In Phase */

dma_xfer_data(dmabyte, 36, 'r');	/* DMA data transfer */

if (scsi_error)

return(0);

cmdcomp();			/* Get Status, Msg, accept & disconnect */

if (scsi_error)

return(0);		/* exit, abort */

while ((status & 0x03) || scsi_stat || cur_data)	/* Poll for Bus Free */

{scsi_stat = inportb(scsi_bus);

status = inportb(stat_reg);

cur_data  = inportb(scsi_data);

}

if (parms.detail == 'y') {

printf("\nID: ");

for (i = 8; i < (dmabyte[4] + 5); i++)

putch(dmabyte[i]);		/* listing the id string */

}

return(0);

}

/********************************* DATA_DRIVER ******************************

DATA_DRIVER (rw, cdb_len, data_len)

input argument rw = phase target is expected to request.r for Data In phase,

w for Data Out.

cdb_len = number of bytes in Command Descripter Block

data_len = number of bytes in data block

This is the driver that calls the four routines that programs the 53C80S to:

Select a device, Transfer data across the SCSI bus, receive Status and

Message bytes from the Target, and disconnect from the SCSI bus.

******************************************************************************/

void DATA_DRIVER(char rw, unsigned short cdb_len, int data_len) {

pause_flag=true;

select(cdb_len);				/* select a device */

if (scsi_error)

return;

while (!(inportb(scsi_bus) & 0x20));	/* Poll for REQ/ active (valid Phase) */

while ((inportb(scsi_bus) & 0x1C) == 0x1C) 		/* Message In ? */

{

Message_In();		/* returns w/ valid phase */

if (scsi_error)

return;

}

if(parms.detail == 'y')

{if (parms.DMA == 'y')					/* DMA data transfer */

printf("\n DMA ");

else

printf("\n nonDMA ");

}

scsi_stat = inportb(scsi_bus);

if (rw == 'w')

{if ((scsi_stat & 0x1C) != 0)           /* Data Out Phase? */

{scsi_error = true;

return;                         /* Illegal Phase, abort */

}

outportb(tgt_cmd, 0);			/* Set PC bits to Data Out */

if(parms.detail == 'y')

printf("Data Out ... ");

}

else

{if ((scsi_stat & 0x1C) != 0x04)        /* Data In Phase? */

{scsi_error = true;

return;                         /* Illegal Phase, abort */

}

outportb(tgt_cmd, 1);			/* Set PC bits to Data In */

if(parms.detail == 'y')

printf("Data In ... ");

}

if (parms.DMA == 'y')					/* DMA data transfer */

dma_xfer_data(&dmabyte[0], data_len, rw);

else 							/* non-DMA data trasfer */

nonDMA_xfer_data(dmabyte, data_len, rw);

if (scsi_error)

return;

while (!(inportb(scsi_bus) & 0x20));	/* Poll for REQ/ active (valid Phase) */

while ((inportb(scsi_bus) & 0x1C) == 0x1C) 		/* Message In ? */

{

Message_In();		/* returns w/ valid phase */

if (scsi_error)

return;

}

cmdcomp();		/* Get Status, Msg, accept & disconnect */

if (scsi_error)

return;		/* exit, abort */

while ((status & 0x03) || scsi_stat || cur_data)	/* Poll for Bus Free */

{scsi_stat = inportb(scsi_bus);

status = inportb(stat_reg);

cur_data  = inportb(scsi_data);

}

}

/********************************** select *********************************/

/* Arbitrates for the bus until wins, Selects, Sends Identify Message Byte if

Selected with ATN/ & Sends CDB */

void select(unsigned short cdb_len) 		  /* Select an SCSI device */

{

scsi_error = false;

outportb(tgt_cmd, 0);

outportb(sel_ena, parms.mid);		/* Enable Selection-Reselection */

outportb(scsi_data, parms.mid);		/* ID bit to Output Data Reg */

/* test         outportb(init_cmd, 0x08);	/* Set BSY/ to test Arb */

arb:	mode = 0x01;

outportb(mod_reg, mode);                /* Begin Arbitration */

/* test		while (!kbhit());

getch();

outportb(init_cmd, 0);	  /* Clear BSY/ to start arb */

while (!(inportb(init_cmd) & 0x40));	/* While Arb in Process = 0 */

/* Wait 2.2 usec (min) Arbitration Delay here */

if (inportb(init_cmd) & 0x20)		/* IF Lost Arbitration */

{mode = 0x00;

outportb(mod_reg, mode);	/* Reset the Arb bit */

goto arb;

}

if ((parms.mid ^ inportb(scsi_data)) > parms.mid)  /* If Higher Id on Bus lose arb (^ bitwise exclusive or to get rid of my ID */

{mode = 0x00;

outportb(mod_reg, mode);	/* Reset the Arb bit */

goto arb;

}

initiat = 0x04;

outportb(init_cmd, initiat);		/* Assert SEL/ (initiat was 0) */

if(parms.atn == 'y')

{initiat = initiat | 0x02;

outportb(init_cmd, initiat);	/* Assert ATN/ */

}

if (inportb(init_cmd) & 0x20)		/* IF Lost Arbitration */

{initiat = 0;

outportb(init_cmd, initiat);	/* Reset SEL/ & ATN/ */

mode = 0x00;

outportb(mod_reg, mode);	/* Reset the Arb bit */

goto arb;

}

/* Wait 1.2 usec (min) Bus Clear & Settle Delay here */

bus_ID = parms.mid | parms.tid;

outportb(scsi_data, bus_ID);		/* Target & Init IDs on bus */

initiat = initiat | 0x09;

outportb(init_cmd, initiat);		/* Assert BSY/ & Data Bus (do not change SEL/ & ATN/ */

mode = mode & 0xFE;

outportb(mod_reg, mode);		/* Reset Arb bit */

initiat = initiat & 0xF7;

outportb(init_cmd, initiat);		/* Reset Assert BSY/ */

printf("\n\n\n Waiting for Selection Interrupt... ");

isr(0);

if (scsi_error)

return;

delay(200);				/* Wait 250ms from BSY deasserted */

/* ( isr(0) used some of this time */

if ((inportb(scsi_bus) & 0x40) != 0x40)

{printf("\n Selection Timeout");

scsi_error = true;

return(1);                      /* Selection timeout, abort */

}

outportb(sel_ena, 0x00);		/* Reset Select Enable Reg */

initiat = initiat & 0xFA;

outportb(init_cmd, initiat); 		/* Deassert SEL/ & Data Bus */

while (!(inportb(scsi_bus) & 0x20));	/* Poll for REQ/ active (valid Phase) */

if(parms.atn == 'y')

{

if(parms.detail == 'y')

printf("\nSelection with ATN ");	/* non-DMA select with ATN */

scsi_stat = inportb(scsi_bus);

if ((scsi_stat & 0x18) != 0x18)		/* Msg Out Phase? */

{scsi_error = true;

return;

}

initiat = 0;

outportb(init_cmd, initiat);		/* Clear ATN/ (this is normally done after ACK is asserted & must be done 90ns before ACK/ is asserted */

outportb(tgt_cmd, 0x06);      		/* Set PC bits = Msg Out */

outportb(scsi_data, parms.msg_byte);	/* Data for Send */

Init_REQ_ACK('w');			/* Send Identify Msg */

}

else

if(parms.detail == 'y')

printf("\nSelection without ATN ");	/* non-DMA select without ATN */

while (!(inportb(scsi_bus) & 0x20));	/* Poll for REQ/ active */

scsi_stat = inportb(scsi_bus);

if ((scsi_stat & 0x08) != 0x08)		/* Command Phase? */

{scsi_error = true;

return;

}

outportb(tgt_cmd, 0x02);      		/* Set Phase Control bits = Command Phase */

if(parms.detail == 'y')

printf("\n Send CDB... ");

for (i=0; i < cdb_len; ++i)

{

outportb(scsi_data, cdb[i]);		/* CDB byte to Send */

Init_REQ_ACK('w');				/* Send CDB byte i */

scsi_stat = inportb(scsi_bus);

if ((scsi_stat & 0x08) != 0x08)		/* Command Phase? */

{scsi_error = true;

return;

}

}

if(parms.detail == 'y')

printf("\ complete");

}									/* end of subroutine select */

/*********************** Initiator REQ/ ACK/ handshake *********************/

void Init_REQ_ACK(char rw)

{

while (((scsi_stat = inportb(scsi_bus)) & 0x20) != 0x20);	/* Poll for REQ/ active */

if (rw == 'w')

{initiat = initiat | 0x01;

outportb(init_cmd, initiat);		/* Assert Data Bus */

}

else

rcv_byte = inportb(scsi_data);		/* Receive byte */

initiat = initiat | 0x10;

outportb(init_cmd, initiat);			/* Assert ACK/ */

while (((scsi_stat = inportb(scsi_bus)) & 0x20) == 0x20);	/* Poll for REQ/ inactive */

initiat = initiat & 0xEE;

outportb(init_cmd, initiat);			/* Deassert ACK & Data Bus */

}

/*************************** DMA Transfer Data ****************************

dma_xfer_data (start_addrs, count, rw)

Input Arguments:

start_addrs = starting addres in memory of data block

count 	    = number of bytes to transfer

rw 	    = direction of data flow.  r for Data In, w for Data Out

This routine will transfer the specified number of bytes across the SCSI

bus.

***********************************************************************/

void dma_xfer_data(char far* start_addrs, int count, char rw)

{

--count;				/* to get EOP w/ last byte */

initIBMdma(start_addrs, count, rw);	/* address, byte count, direction */

if (rw == 'w')

{

mode = 0x02;		    	/* DMA Mode */

if (parms.ena_eop == 'y')

mode = mode | 0x08;	/* Enable EOP Int */

outportb(mod_reg, mode);

/* Set PC bits prior to calling dma_xfer_data */

initiat = initiat | 0x01;

outportb(init_cmd, initiat);	/* Assert Data Bus */

outportb(dma_send, 0);			/* Start DMA Send */

while (!(inportb(tgt_cmd) & 0x80));     /* Poll for Last byte sent */

}

else

{

mode = 0x02;            	/* DMA Mode */

if (parms.ena_eop == 'y')

mode = mode | 0x08;	/* Enable EOP Int */

if (parms.parity == 'y')

mode = mode | 0x30;	/* Enable Parity chking & Int */

outportb(mod_reg, mode);

/* Set PC bits prior to calling dma_xfer_data */

outportb(dma_init_rcv, 0);	/* Start DMA Receive */

}

isr(2);					/* EOP or End of DMA Interrupt */

if (scsi_error)

return;

mode = 0;

outportb(mod_reg, mode);		/* Reset DMA & other mode bits */

initiat = initiat & 0xFE;

outportb(init_cmd, initiat);		/* Reset Assert Data Bus */

if(parms.detail == 'y')

printf("complete");

}

/************************* non-DMA Transfer Data *************************/

void nonDMA_xfer_data(char far* start_addrs, int count, char rw)

{

unsigned int out_count;

out_count = 0;

while (out_count < count)

{scsi_stat = inportb(scsi_bus);

if (rw == 'w')

{if ((scsi_stat & 0x1C) != 0)

{scsi_error = true;

return;

}

/* Set PC bits prior to calling nonDMA_xfer_data */

outportb(scsi_data, start_addrs[out_count]);

Init_REQ_ACK(rw);

++out_count;

}

else

{if ((scsi_stat & 0x1C) != 4)

{scsi_error = true;

return;

}

if (parms.parity == 'y')

mode = 0x30;		/* Enable Parity chking & Int */

outportb(mod_reg, mode);

/* Set PC bits prior to calling nonDMA_xfer_data */

Init_REQ_ACK(rw);

start_addrs[out_count] = rcv_byte;

++out_count;

}

}	/* End while */

mode = 0;

outportb(mod_reg, mode);		/* Reset mode bits */

initiat = initiat & 0xFE;

outportb(init_cmd, initiat);		/* Reset Assert Data Bus */

if(parms.detail == 'y')

printf("complete");

}

/******************************** cmdcomp **********************************/

void cmdcomp(void) {				/* Get Status, Msg, accept & disconnect*/

statbyte = 0xFF;

msg = 0xFF;

if(parms.detail == 'y')

printf("\nStatus and Message ... ");

while (!(inportb(scsi_bus) & 0x20));	/* Poll for REQ/ active */

scsi_stat = inportb(scsi_bus);

if ((scsi_stat & 0x1C) != 0x0C)		/* expect Status Phase */

{printf("\n ERROR! Current SCSI Bus Status = %02X (expected 4C)", scsi_stat);

scsi_error = true;

return;                         /* abort, Illegal Phase */

}

outportb(tgt_cmd, 0x03);		/* Set PC bits to Status Phase */

Init_REQ_ACK('r');

statbyte = rcv_byte;

while (!(inportb(scsi_bus) & 0x20));	/* Poll for REQ/ active */

scsi_stat = inportb(scsi_bus);

if ((scsi_stat & 0x1C) != 0x1C)		/* expect Msg In Phase */

{printf("\n ERROR! Current SCSI Bus Status = %02X (expected 5C)", scsi_stat);

scsi_error = true;

return;                         /* abort, Illegal Phase */

}

outportb(tgt_cmd, 0x07);		/* Set PC bits to Msg In Phase */

Init_REQ_ACK('r');			/* Get & accept Msg byte */

msg = rcv_byte;

if (statbyte == 0 && msg == 0)

if(parms.detail == 'y')

printf("good");

else if (statbyte == 2 && msg == 0)

if(parms.detail == 'y')

printf("status = 02 CHECK CONDITION");

else

{scsi_error = true;

return;

}

}

/********************************* rqsense **********************************

rqsense(cdb_len)

input argurment cdb_len = length of the command descripter block

This routine is a driver for the Request Sense SCSI command

***************/

void rqsense(unsigned short cdb_len) {

pause_flag = true;

select(cdb_len);			/* select a device */

if(scsi_error)

return;				/* exit, abort */

temp = parms.ena_eop;			/* save current ena EOP int setting */

parms.ena_eop = 'y';			/* must depend on EOP int since

the phase may not change */

while (!(inportb(scsi_bus) & 0x20));	/* Poll for REQ/ active */

scsi_stat = inportb(scsi_bus);

if ((scsi_stat & 0x1C) != 0x04)         /* Data In Phase? */

{scsi_error = true;

return;				/* Illegal Phase, abort */

}

printf("\n Request Sence Data In ...");

outportb(tgt_cmd, 1);			/* Set PC bit to Data In Phase */

dma_xfer_data(&dmabyte[0], 8, 'r');	/* DMA Data In, 8 bytes */

parms.ena_eop = temp;			/* restore ena EOP int setting to orig */

if (parms.ena_eop == 'y')

mode = mode | 0x08;		/* Set to original (Enable EOP Int) */

outportb(mod_reg, mode);

if(scsi_error)

return;				/* exit, abort */

cmdcomp();				/* Get Status, Msg, accept & disconnect */

if (scsi_error)

return;				/* exit, abort */

while ((status & 0x03) || scsi_stat || cur_data)	/* Poll for Bus Free */

{scsi_stat = inportb(scsi_bus);

status = inportb(stat_reg);

cur_data  = inportb(scsi_data);

}

}

/*********************** Message In after Command Phase *********************

DisConnect then ReSelect

Message_In()

input argument:	rw = data direction, r (Read) for Data In, w for Data Out

*************/

void Message_In()

{

if(parms.detail == 'y')

printf("\nMessage In ... ");

outportb(tgt_cmd, 0x07);

temp1 = inportb(scsi_data);				/* Receive byte */

initiat = 0x10;

outportb(init_cmd, initiat);				/* Assert ACK/ */

while (((scsi_stat = inportb(scsi_bus)) & 0x20) == 0x20);	/* Poll for REQ/ inactive */

switch (temp1) {			/* temp1 = message-byte */

case 0x02 :

if (parms.detail == 'y')

printf(" ... Save Data Pointers");

initiat = 0;

outportb(init_cmd, initiat);	/* Clear ACK for msg accept */

break;

case 0x04 :

if (parms.detail == 'y')

printf(" ... Disconnect Message");

initiat = 0;

outportb(init_cmd, initiat);	/* Clear ACK for msg accept */

while ((status & 0x03) || scsi_stat || cur_data)	/* Poll for Bus Free */

{scsi_stat = inportb(scsi_bus);

status = inportb(stat_reg);

cur_data  = inportb(scsi_data);

}

outportb(sel_ena, parms.mid);	/* enable ReSelection */

printf("\n\n\n Waiting for Reselection Interrupt");

isr(1);				/* wait for ReSelect */

if (scsi_error)

return;

bus_ID = cur_data;

while (((scsi_stat = inportb(scsi_bus)) & 0x20) != 0x20);	/* Poll for REQ/ active */

ID_msg = inportb(scsi_data);					/* Receive byte */

initiat = 0x10;

outportb(init_cmd, initiat);					/* Assert ACK/ */

while (((scsi_stat = inportb(scsi_bus)) & 0x20) == 0x20);	/* Poll for REQ/ inactive */

if (ID_msg != 0x80)						/* Identify Message Bad */

{printf("\n unsupported Identify Message");

scsi_error = true;

return;

}

initiat = 0;

outportb(init_cmd, initiat);	/* Clear ACK for msg accept */

break;

default:

printf(" ... received unsupported message = %02X hex",temp1);

scsi_error = true;

return;

}	/* end of switch() */

while (((scsi_stat = inportb(scsi_bus)) & 0x20) != 0x20);	/* Poll for REQ/ active */

/* Phase valid */

}	/* end of Message_In() */

/*************************** Read 53C80S Registers **********************

This routine reads the register in the 53CF92

***********/

void read_regs(void)

{

clrscr();

printf("\nThe current 53C80S register contents are: \n");

printf("\n00  Current SCSI Data                =   %02X",inportb(scsi_data));

printf("\n01  Initiator Command                =   %02X",inportb(init_cmd));

printf("\n02  Mode                             =   %02X",inportb(mod_reg));

printf("\n03  Target Command                   =   %02X",inportb(tgt_cmd));

printf("\n04  Current SCSI Bus Status          =   %02X",inportb(scsi_bus));

printf("\n05  Bus & Status                     =   %02X",inportb(stat_reg));

printf("\n06  Input Data                       =   %02X",inportb(in_data));

#

printf("\n\nHit any key to return to menu");

temp = getche();

} 									/* end of read_regs() */