## Windows 95 FAQ

- Q: Where can I find drivers for the MGA Millennium?

- A: There is a driver on the Windows 95 CD ROM in the \DRIVERS\DISPLAY\MATROX directory. If you did not receive Windows 95 drivers with your Millennium, you can also obtain our Powerdesk drivers for Windows 95 from our BBS or from Compuserve.

- Q: Where can I find drivers for other MGA products?

- A: Drivers for other MGA products are included with Windows 95 and are installed automatically.

- Q: Are the PowerDesk for Windows 3.1 features compatible with Windows 95?

- A: No. After upgrading from Windows 3.1 to Windows 95, you cannot use the programs in the Windows 3.1 PowerDesk group. We actually recommend that you uninstall the MGA software prior to upgrading to Windows 95. If you install Windows 95 over Windows without first uninstalling the old MGA software, you should delete the PowerDesk group folder and all its icons, then install the Windows 95 version of the MGA software.

- Q: I start Windows 95 and my monitor does not sync properly. What can I do?

- A: This probably happens because the MGA driver is trying to boot in a resolution that your monitor does not support. This could occur if you had never selected a monitor, or if you physically changed your monitor without informing Windows 95 (or the MGA Monitor Program).

To recover from this problem, reboot your system (by pressing Ctrl-Alt-Del twice). When the screen displays "Starting Windows 95", press F5 to enter Windows 95 in "safe mode". In this mode, Windows 95 will boot in VGA at 640 x 480, and you will then be able to lower the resolution in the Display Properties dialog box. Pull it down to 640 x 480 and reboot again to get a display of 640 x 480 in Millennium mode. Then take the appropriate actions to check and correct your monitor type.

- Q: I have a problem with an application that I suspect is caused by the display driver. What should I do?

- A: You can try using the application with the standard VGA driver, or alternately, try using the Millennium driver, but with hardware accelerations disabled (see below).

To restart in VGA mode, simply open the Display Properties  dialog box (right-click on the desktop background), and change the Display Type to VGA. You will have to reboot.

To restart using the Millennium driver with hardware accelerations off, click on the System application in the Windows 95 Control Panel, then go to the Performance page. Select Graphics, then move the Hardware Acceleration bar to None. You will have to reboot as well, but you will still have the same Display area and Color Palette. This is often better than using standard VGA, as some applications exhibit problems, specifically in High Color or True Color modes.

- Q: Windows 95 reports a display adapter hardware conflict upon booting. I thought Windows 95 was Plug and Play! What can I do?

- A: This does sometimes happen. If you have changed display adapters, especially non-PCI ones, you should let the Windows 95 hardware detection program take the appropriate steps as suggested; this might solve your problem.

If you have not changed any adapter, running this program is of no use. Instead, run the System application in the Control Panel, then select Device Manager, then Display Adapters. If you can't solve the problem here (or if no problem is reported), try re-installing the Millennium driver from the original diskette. To do this, open the Display Properties  window, click on Change Display Type, then re-select the MGA Millennium.

- Q: Can I use the Millennium Multimedia Superpack CD-ROM under Windows 95?

- A: The versions of 3D-FX and the MPEG player included with version 1.0 of the SuperPack do not work correctly with Windows 95. Updates for 3D-FX may be downloaded from our BBS or WWW/FTP site. Updates for the MPEG player will be available at a later date.

- Q: Can I uninstall the MGA PowerDesk for Windows 95 software?

- A: Yes, but not by using the standard Windows 95 Install/Remove Applications in Control Panel, since PowerDesk is so fully integrated with the display driver. Here is what you must do (this should only be attempted by advanced users):

- Make sure you are not using a Desktop area that is larger than the Display area (look on the MGA Settings  sheet in the Display Properties dialog box).

- Run \Matrox\Millen\Paneloff.reg to remove the MGA Settings page.

- Edit \windows\win.ini  to remove mgactrl.exe from the load= line.

- Remove the MGA Millennium PowerDesk folder from the Programs flyout of the Start menu, then delete the \Matrox directory structure.