tMidi.doc                                   Copyright (C)1997 Cornel Huth

Release 2.10a                                                 10-Apr-1997

tMidi is a real-time, VIO-based standard MIDI file player.  It also can

be used with an attached keyboard to completely control the MPU(s).  You can

play an SMF and the keyboard at the same time, even with just one MPU.

Files included are:

1. paradise.sys

2. tmidi.exe

3. several SMFs, documentation

Optionally, if you are running Warp 3 or earlier:

4. clock01.sys  (clock02.sys available by request, for MCA)

No other drivers are needed.  MMPM/2 is not required (but is okay), nor

are any soundcard drivers.  This package includes everything you need for

perfect MIDI playback, even under heavy multitasking.

For related files go to http://www.40th.com/ and look around the Paradise

section.  If you are using an older version of the Paradise driver, be

sure to replace it with the driver in this package (version 1.6).

Installation

------------

Install the (two) SYS driver(s) in your boot drive's root directory and

include a reference for the paradise driver in your config.sys:

DEVICE=G:\PARADISE.SYS     (optional -q for quiet install)

Paradise version 1.6 or later is required.

CLOCK01.SYS, installed automatically by the boot process and so has no

reference in the config.sys file, is based on the Warp 4.0 driver.  It

can be used in previous OS/2 versions, too.  If you are using Warp 4.0,

your CLOCK driver already has the IDC hook, so you can use the one you

already have.  The included CLOCK01.SYS driver is only for those running

Warp 3 or earlier.

If you have the old clock driver installed, paradise.sys will gracefully

tell you so, and not install itself.  If you get this message at boot, use

the included clock01.sys.

Note:  Due to a long-standing bug in OS/2, detection of obsolete clock

drivers is not as straight-forward as it should be.  However, if the

obsolete clock is not found during boot, it likely will be found by tmidi

(tmixer doesn't current make use of the clock driver, but the Paradise

system requires that it be installed anyway).  See tmidi.doc for more.

The clock01.sys driver in Warp 3's fixpack #26 is -not- an up-to-date

driver.  If the clock01.sys driver is substantially less than 4000 bytes

(e.g., 3520 bytes, like the fp26 one (date 10-10-96)) you need to install

the clock01.sys driver included in this package.

Once you have the driver(s) installed, reboot.

Note:  There is no need to remove any driver from config.sys.  All you

need to do is add an entry for PARADISE.SYS, and possibly copy the

clock01.sys included in this package to boot:\.  That's all there is

to it.

Using tMidi

-----------

For a detailed technical document in the use of tMidi, register using the

included _order.* forms.  This evaluation version contains basic setup and

use instructions, which is enough to see whether you want to use it.

Use no args for a list of switches:

[F:\mids\fine]tmidi

tMidi 2.10   Copyright (C)1997 Cornel Huth   http://www.40th.com/

Use: [G:\]tmidi [-switch] file.mid

-p1:x     x=hex baseport of MPU for port 1  (e.g., -p1:330 [def])

-p2:x     x=hex baseport of MPU for port 2+ (e.g., -p2:300)

-gm       send GM ON sysex before play

-gs       send GS ON sysex before play

-xg       send XG ON sysex before play

-tm:x     x=hex track mask, to play selected tracks only (x=1-FFFFFFFF)

-tk:n     n=track to mask, repeat as required (n=1 to 32)

-w1:n     n=max timeout for port 1, n=1000 to 65535, def=10000

-w2:n     n=max timeout for port 2, n=1000 to 65535, def=10000

"-sx1:x x"       x=hex byte(s) to output to -p1: (single ch msg or sysex)

"-sx2:x x"       x=hex byte(s) to output to -p2: (must be in "marks")

"-kb:i pi ch po" IRQ,portIn=1|2,channel=1-16,portOut=1|2 ("-kb:10 1 16 1")

The default MPU port is 330h, and if there, does not need to be specified.

The Paradise driver handles up any number of ports, and any number of

devices, though tMidi currently supports 16 ports and 2 device.  If a

second device is specified, all output to ports 2 through 16 go to it.

An example output during a run:

[F:\paradise\tmidi\mids]tmidi -p1:330 -p2:300 -gm -tm:FFFFF inferno.mid

1:45  fmt:1 tks: 9 div:120  GM    123456789 1234567890

0:15.88  6/8   4:1  67bpm  136Hz  TTTTTTTTT PP.......P

In this case, 3 ports have been specified:  port 1 data goes to the MPU

at -p1:330; port 2 and port 10 data goes to the MPU at -p2:300.  All

tracks are played (-tm:FFFFF used only for example).  The screen output

makes use of ANSI color, where it's needed.

For details, see tmidi.tec (registered version).

Specifying Mode

---------------

An optional switch is the MIDI playback mode:  GM, GS, or XG.  If the

SMF contains no embedded mode, you can specifiy it via the command line.

If the SMF already contains a mode, you cannot override it.

Specifying Tracks

-----------------

Supply either a hex mask for tracks to play, or select them individually

using -tk:.  Only the first 32 tracks can be played if either of these

are used, otherwise, 1000+ tracks can be played.

Specifying Waits

----------------

To detect when the MPU interface has been reset out from under it, the

Paradise driver checks for a timeout condition to occur.  By default, it

counts down 10,000 times before concluding that something's gone wrong.

For very fast systems this may be too soon.  If you get a time-out error

reported, or otherwise do not hear anything when you expect you should,

use the -w1: switch to set this wait timeout value.  The valid range is

1000 to 65535, with the default set to 10,000.

An additional fail-safe has been added to this time-out check:  when a

time-out condition has occured 5 times in a row the current play is

stopped and tmidi exits.  The next play should go as expected.

Specifying Data to Send To the MPU

----------------------------------

Using -sx1: (-sx2: for -p2: MPU) you can send a single channel or system

exclusive messages to the MPU(s).  The entire argument must be enclosed in

double-quote marks.  For example:

[G:\]tmidi -gs "-sx1:F0 41 42 12 40 00 7F 00 FF F7"

The above sends a GS RESET sysex message to the MPU.  The checksum is

calculated if FF is used in the checksum place (and -gs).  See tmidi.tec

(available to registered usrs only) for details and tips.

Specifying an External Controller

---------------------------------

If you have an external controller, like a MIDI-capable keyboard, you can

use it with tMidi.  You can specify the IRQ (required), portIn, portOut,

and MIDI channel to output to.  See tmidi.tec for details and tips.  The

entire argument must be enclosed in double-quote marks.

You must supply either a SMF filename or a sysex/channel msg (-sx above) on

the command line with this switch.

For example, to play the keyboard connected to the port 1 device, using

IRQ 10, out to the port 1 device, on channel 16:

[G:\]tmidi xg_on.mid           ;switch to XG mode

[G:\]tmidi "-sx1:BF 00 40"     ;switch to SFX bank (may need BF 20 00 also)

[G:\]tmidi "-sx1:CF 21" "-kb:10 1 16 1"  ;set program to THUNDER, then

;using IRQ10, read from port 1 device

;and output to channel 16 on port 1 device

When you press a key, you get rolling thunder!

The keyboard remains active until you press a key on the (type) keyboard,

such as ENTER (or up-arrow).

Limitations

-----------

A compatible MPU-401 device is required.  This includes not only

real MPU-401 devices, but cards like the Sound Blaster 16, AWE32, etc.,

with either a daughterboard (CL Wave Blaster, Yamaha DB50XG, etc.), or

a sound module or keyboard attached to the external MIDI port.  Stand-

alone cards, like the Yamaha SW60XG, can be used, too.  There are no

plans to work with other, non-MPU devices at this time.

Tips

----

See tmidi.tec.

See the README for more.

Filename is tmidi210.zip.