aMixer.doc                                  Copyright (C)1997 Cornel Huth

release 1.0                                                   26-Mar-1997

aMixer is a full-function mixer for Creative Labs Sound Blaster soundcards.

Controls include volume, balance, bass, treble, more levels and balances,

gain in and out, input and output connections, AGC switch, save/load mixer

settings, and SyncLock(tm) to automatically program the mixer with your

settings at regular intervals as short as 33ms.

aMixer is an OS/2 PM application (MMPM/2 is not required).  Files included:

1. paradise.sys

2. amixer.exe

Optionally, if you are running Warp 3 or earlier:

3. clock01.sys  (clock02.sys available by request, for MCA)

No other drivers are needed.  MMPM/2 is not required (but is okay), nor

are any soundcard drivers.  This package includes everything you need

to control your SB mixer.  Specific mixers currently supported are the

1335 (SB2CD), 1345 (SB Pro), and the 1745 (SB16 and later).

For related files go to http://www.40th.com/ and look around the Paradise

section.  If you are using an older version of the Paradise driver, be

sure to replace it with the driver in this package (version 1.5).

Installation

------------

Install the (two) SYS driver(s) in your boot drive's root directory and

include a reference for the paradise driver in your config.sys:

DEVICE=G:\PARADISE.SYS     (optional -q for quiet install)

Paradise version 1.5 or later is required.

CLOCK01.SYS, installed automatically by the boot process and so has no

reference in the config.sys file, is based on the Warp 4.0 driver.  It

can be used in previous OS/2 versions, too.  If you are using Warp 4.0,

your CLOCK driver already has the IDC hook, so you can use the one you

already have.  The included CLOCK01.SYS driver is only for those running

Warp 3 or earlier.

If you have the old clock driver installed, paradise.sys will gracefully

tell you so, and not install itself.  If you get this message at boot, use

the included clock01.sys.

Note:  Due to a long-standing bug in OS/2, detection of obsolete clock

drivers is not as straight-forward as it should be.  However, if the

obsolete clock is not found during boot, it likely will be found by tmidi

(tmixer doesn't current make use of the clock driver, but the Paradise

system requires that it be installed anyway).  See tmidi.doc for more.

The clock01.sys driver in Warp 3's fixpack #26 is -not- an up-to-date

driver.  If the clock01.sys driver is substantially less than 4000 bytes

(e.g., 3520 bytes, like the fp26 one (date 10-10-96)) you need to install

the clock01.sys driver included in this package.

Once you have the driver(s) installed, reboot.

Using aMixer

------------

First, aMixer was designed primarily for the 1745 mixer (SB16 and later).

If you are using an SB Pro or SB2CD, then many of the controls do not

apply.  They have not been disabled for aesthetic reasons.  If you have

an SB Pro, be aware that your mixer (1345) does not have Bass, Treble,

Speaker, or Gain In/Out, nor does it have any output connections.  You

can use the controls, but there won't be an effect.  The SB2CD (1335)

has fewer controls, still (only volumes: main, wave, MIDI, CD).

The aMixer.ini File

When you first use aMixer a configuration menu is shown.  Select your

base port, IRQ, DMA, and mixer (Auto is recommended).  All that is needed

is the base port (usually 220h).  In order for aMixer to be able to reset

the SB16, you need to supply the correct IRQ and DMA channel(s).  This only

needs to be once, when you first use aMixer.  You can later change these

settings by starting aMixer with the -config switch.  aMixer stores this

information in the amixer.ini file, which is located in the same directory

as the directory from which amixer.exe is executed.  If this directory is

not writable, you cannot save any settings.  If you delete the ini file,

you will be presented with the configuration menu again,  If you ever have

a problem with aMixer you may want to delete the amixer.ini file and go

again.

Main Control Panel

The main control panel of aMixer consists of the Master Volume and Balance,

Bass and Treble.  Also on this panel are About and Detail buttons.  Pushing

About pops up a box showing the current version, and other key information.

Pushing the Detail button brings up the Detail Control Panel.

Detail Control Panel

The title of this panel identifies the mixer (1335, 1345, or 1745).  Like

the Main Control Panel, this panel has volumes and balances, and more

controls that set connections, and save/load/reset mixer settings.  If you

are concerned about the way the control knobs jump around read the

Limitations section below.

The Connections are for the input and output (I/O) pathways.  A check next

to the I indicates that that pathway is connected for input.  If the O is

checked, then it indicates that that pathway is also connected to the

output.  An input checked means that you can record from that pathway.

An output checked means that that pathway is connected to the output.  For

example, if Mic output is checked, you can speak into the mic and have the

sound routed to the output -- a mini PA system.  Controls without an I or O

option are not switchable (they are always connected).

If AGC is check, the automatic gain control circuit is activated for mic

input.  The AGC is a sort of limiter.  This makes it easy to record sources

that vary widely in dynamic range, allowing you to capture low sound levels

and still not overload the recording circuits at high sound levels, even

without having to actively control the mic level.  When AGC is unchecked,

the mic circuit is set to +20dB of gain.  For less noise, I recommend that

the mic input and output be unchecked unless you are using them.  This goes

for all connections, for that matter.  Also set their levels to -0-.

The Save button saves the current mixer state as known to aMixer to the

amixer.ini file.  This is the only time, other than when it is first

created, that any settings are saved.  At startup the amixer.ini is read

and the mixer settings in amixer.ini are programmed into the mixer.  To

prevent this, use -noload (see Tips, below).

The Load button loads the settings from the amixer.ini file and programs

the mixer using these settings.  This includes all settings that aMixer

uses, including SyncLock settings.

The Startup button programs the mixer using the settings that were in force

at program startup.  Unless the -noload was used, these settings are those

that were in the amixer.ini file when aMixer started.

The Reset button resets the DSP and programs the mixer to reset (to default

settings), if possible.

The SyncLock Config button brings up the configuration menu for SyncLock.

SyncLock(tm) Configuration Panel

The SyncLock feature of aMixer programs the mixer at a set rate, from once

a second up to 33 times per second.  This feature is useful if you are using

MMPM/2 since MMPM/2 resets the mixer each and every time it is used by an

app.  Even system sounds will mess with your mixer volumes and settings, and

this is especially bothersome since it happens all the time.

Select the settings that you want to be SyncLock'ed, and the rate at which you

want aMixer to do this.  To disable SyncLock, set the Sync Rate to -0-.  The

minimum active rate is 33ms (milliseconds), or about 33 times per second.  You

may want to set the rate to something less frequent, like 100ms, in which case

the mixer is restored to your settings 10 times a second.  The more frequently

aMixer restores the mixer then less likely you will notice MMPM/2 messing with

your settings.

I prefer to check all volumes, and mic in and out, and leave the rest (the

other connections) unchecked.  This way I control the levels at all times.

The settings here takes affect as soon as you press the OK button.

Limitations

-----------

The 1745 connections are always programmed left-to-left, and right-to-right

(e.g., Line-In left always goes to the left channel, and Line-In right

always goes to the right channel).

The gain controls are programmed to adjust both channels identically (e.g.,

setting gIn:50 sets both left and right gain-in channels to 50).  Ditto

for the tone controls.

Volume levels are not continuous; when you move a control, it will jump to

the nearest level that your sound card mixer actually has.  As an example,

the Gain Out control has only four positions, and it will jump from 0, to

40, to 65, to 100.

To increase the Gain Out level you must hold down either Shift key, or the

left Control key.  You can decrease this level without doing this.

For the 1345 (SB Pro), when selecting the input source, you may need to

uncheck CD before you can select Line.  Only one should be checked (only

one can be active at any time).  If more than one is checked (mic, CD, line),

uncheck all and then check the one you want as the input source.  Note that

the mic In can only be unchecked by selecting either CD or Line In; the

Mic will be the input by default when neither CD or Line is selected.

One instance per path\amixer.exe is allowed.  If you ever have a need to

run more than one aMixer, use another path to locate the additional EXE(s).

It is not recommended to have two aMixers running against the same mixer,

at the same base port.

Tips

----

The following optional switches may be used on the command line when starting

aMixer:

[G:\]aMixer -config -beep -reset -noload

-config        Brings up the configuration menu to select base port, IRQ,

DMA channel(s), and mixer.  You may also get this by deleting

the amixer.ini file, located in the directory of amixer.exe;

this is automatic if no amixer.ini exists in the directory

of where the amixer.exe file resides.

-beep          Beeps now and then, for audible feedback.  Low frequency

(100Hz, 1 second long) indicates a problem (usually not

fatal).  Short, high frequency beeps (1760Hz, 66ms) are

just feedback when some buttons are pushed (save/load,

etc.).

-reset         When used, and when the IRQ and DMA channel(s) have been

provided, the SB DSP is reset at program start.  This

also resets the mixer.

-noload        To prevent the saved mixer settings in amixer.ini from

being programmed into the mixer at start, use this.

See the README file for more.

Cornel Huth

--

.                                             .

:    Quality Toolkits and Applications for    : info@40th.com

:          DOS - Win16 - Win32 - OS/2         : http://www.40th.com/

:                                             : BBS:1-210-684-8065